# Installation Sheet - Item 1

Customer: REW	Offer No: 4195A	Group/Well: No.

Country: Czech Rebublic	Quantity: 1-6 set	Production: 200 m³/day

| 2.0 | SBS Drive Head | SBS DD KB II 1.1 |
| --- | --- | --- |
| 2.1.1 | Ex-proof Motor | 55 KW/1/ 400V |
| 2.2.2 2.2.5 | Power Cab Stiering C. | 4x25mm²x15m 2x0,75mm²x15m |
| 2.1.7 | 2nd Stuff Box | none |
| 2.1.8 | BOP | BOP / Flow-T |
| 2.1.6 | Adapter Drive Head | none |
|  | 3-1/2 EU #9,2 | none |
| 3.7 | Circulation Valve | none |
|  | 3-1/2 EU #9,2 | none |
| 3.2 | Adapter Pup Joint | none |
| 3.1 | Pup Joint | none |
| 3.3 | Adapter Stator | none |
| 1.2 & 1.3 | PCP Stator with Stop Pin | SBS 120-060 (60-1200) 3-1/2” Box / 3-1/2” Pin |
| 3.3 | Adapter Torque Anchor | ? |
| 3.5 | Packer- Torque Anchor | 7"x 3-1/2"EU Box / Pin discussion |
| 3.6 | Gas Anchor | none |

| 4.1 | Lift-Coupling | PR 1"FS |
| --- | --- | --- |
| 2.1.5 | Polish Rod Clamp | 1 1/2" |
| 4.2 | Polished Rod | 1 1/2" x 16 ft |
| 4.3 | Polished Rod Coupling | PR 1"FS |
| 4.4 | Pony Rods | 1 Set Grade D SBS S2H 1"x2,3,4,6,8,10ft |
| 4.6 | Non Rotating Centralizer | 20 pcs 1" x 3-1/2" |
| 4.5 | Sucker Rod | Pump Setting Depth ~1200 m Grade D SBS S2H 158 pcs 1" x 25 ft |
| 4.7 | Coupling | 1” x 1-1/8” SH |
| 1.1 | PCP Rotor | SBS 120-060 (60-1200) |
| 3.2.5 | Switch Box 55 KW/ 400V 30-55 Hz (241-444 rpm) |  |

Spare Sheave: (Direct driven, expected RPM 350 – 420)	Spare Belt: none